# Texas Title V Air Quality Compliance – Comprehensive Process Guide

Version: December 1, 2025 | Prepared for interactive AI/RAG automation with human-in-the-loop

# PART 1: APPLICABILITY DETERMINATION

## 1.1 Major Source Thresholds (40 CFR 70.2; 30 TAC 122.10)

### 1.1.1 Calculate Potential to Emit (PTE)

Definition: Maximum capacity to emit a pollutant under physical and operational design.

Time basis: tons per year (tpy).

Formula: PTE = Maximum Rate × Hours of Operation × Conversion Factor.

### 1.1.2 Major Source Criteria

Attainment Areas:

100 tpy of any criteria pollutant

10 tpy of any single HAP

25 tpy of total HAPs

Nonattainment Areas (varies by pollutant/severity):

Ozone (Serious): 50 tpy VOC or NOx

Ozone (Severe): 25 tpy VOC or NOx

Ozone (Extreme): 10 tpy VOC or NOx

PM-10 (Serious): 70 tpy

CO (Serious): 50 tpy

### 1.1.3 Other Title V Sources

Subject to NSPS (40 CFR Part 60)

Subject to NESHAP (40 CFR Parts 61, 63)

Affected source under Acid Rain Program (40 CFR Part 72)

Solid waste incineration units (certain 40 CFR Part 60 subparts)

## 1.2 Emission Unit Inventory

Identify all emission units (combustion, process equipment, tanks, loading, fugitives, emergency equipment).

For each unit, capture equipment type/specs, fuel/type/quality, maximum capacity/throughput, operating hours, control equipment.

## 1.3 Emissions Calculations

Use data in the following hierarchy: CEMS, source tests, material balance, AP-42 factors, manufacturer data.

### Example – Natural Gas Boiler

Heat Input: 100 MMBtu/hr; Operating Hours: 8,760 hr/yr; NOx EF: 0.1 lb/MMBtu. NOx Emissions = 43.8 tpy.

### 1.3.2 Storage Tanks

Use AP-42 Chapter 7 methods or approved software. Consider standing/working/breathing losses or floating-roof losses.

### 1.3.3 Fugitive Emissions

Use EPA leak protocols; component counts by service and LDAR level.

## 1.4 Exemptions and Exclusions

### 1.4.1 Synthetic Minor Source (30 TAC 122.120)

Use federally enforceable limits below major thresholds with monitoring, recordkeeping, and reporting.

### 1.4.2 Insignificant Activities (30 TAC 122.120)

Categorically insignificant or de minimis activities must still be identified when relevant.

# PART 2: APPLICABLE REQUIREMENTS IDENTIFICATION

## 2.1 Federal Requirements Matrix

NAAQS context for criteria pollutants

NSPS (Part 60) subparts including A, Dc/Db, IIII/JJJJ, Kb, VVa (as applicable)

NESHAP (Part 63) including ZZZZ (RICE), DDDDD/JJJJJJ (boilers)

PSD/Nonattainment NSR triggers and BACT/LAER

Acid Rain (Part 72), where applicable

## 2.2 Texas State Requirements

30 TAC Chapter 101 (general rules)

Chapter 106 (Permits by Rule)

Chapter 111 (Visible emissions)

Chapter 112 (Sulfur compounds)

Chapter 115 (VOCs)

Chapter 116 (NSR)

Chapter 117 (NOx)

## 2.3 Local/Regional Requirements

Check county/local rules and regional haze requirements.

Tag each requirement as “federally enforceable” or “state-only.”

Screen each controlled unit for CAM (40 CFR part 64).

# PART 3: APPLICATION PREPARATION

## 3.1 Pre-Application Activities

Optional pre-application meeting with TCEQ APD.

Data collection: site plan, PFDs, specs, fuels, SDS, control device info, prior permits/tests, property info.

## 3.2 Prepare Title V Materials (SOP or GOP Registration)

Complete SOP package (or GOP eligibility/registration) and maintain NSR PI-1/technical attachments for incorporated authorizations.

## 3.3 Technical Attachments

Attachment A – Detailed emission calculations with step-by-step and references.

Attachment B – Process flow diagrams with control/monitoring points.

Attachment C – Site layout/plot plan with EPN coordinates.

Attachment D – Equipment/control specifications.

Attachment E – Fuel analyses.

Attachment F – Compliance demonstrations (by rule citation).

Attachment G – BACT analysis (if PSD).

Attachment H – Ambient impact analysis (if required).

## 3.4 Special Demonstrations

GEP stack height analysis.

Netting analysis for modifications (5-year contemporaneous window).

Synthetic minor limit structure (limit + monitoring + recordkeeping + reporting).

## 3.5 Monitoring/Recordkeeping/Reporting Plan

Monitoring: CEMS/CPMS, periodic/parametric, VE observations, LDAR.

Recordkeeping: minimum 5-year retention (Title V).

Reporting: semiannual monitoring, annual certification, prompt deviation, excess emissions.

## 3.6 Fees

Track current TCEQ application and annual emissions fee rates; calculate and document payment.

# PART 4: APPLICATION SUBMITTAL

## 4.1 Completeness Review

Checklist for forms, attachments, signatures, fees, copies.

Certification statement by Responsible Official.

## 4.2 Submittal Methods

Electronic submittal via TCEQ ePermits (preferred).

Paper submittal (as required): TCEQ, EPA Region 6 copies.

## 4.3 Administrative Completeness (45 days)

TCEQ completeness determination or NOD.

Declaration of Completeness starts the technical review clock; Title V must be issued within 18 months of a complete application.

# PART 5: PERMIT REVIEW AND ISSUANCE

## 5.1 Technical Review

Review of calculations, requirements, compliance demonstrations, monitoring adequacy, BACT/ambient analysis where applicable.

RAIs and responses; Draft permit preparation (conditions, monitoring, testing, recordkeeping, reporting, schedules).

## 5.2 Public Notice and Comment (Title V)

30-day public notice in local paper and online; content includes facility info, summary of emissions, how to comment.

Public meeting if requested; formal comment record and response to comments.

## 5.3 EPA Review (Title V)

EPA 45-day review; potential objection and required resolution.

Petition to object within 60 days of close of public comment.

## 5.4 Final Permit Issuance

Standard and site-specific conditions with limits, monitoring, testing, recordkeeping, reporting, and compliance schedules.

Term: up to 5 years; apply for renewal at least 6 months before expiration; application shield applies.

## 5.5 Appeal Rights

Contested case hearing request window (typically 30 days).

SOAH process; commissioners issue final decision.

# PART 6: ONGOING COMPLIANCE

## 6.1 Startup, Shutdown, Malfunction (SSM)

Maintain SSM plan if required; follow permit and rule-specific timing for notifications and written reports.

Perform root-cause analyses and document corrective/preventive actions.

## 6.2 Emission Monitoring

CEMS/Part 75 where required; QA/QC, certification, RATAs, data validation, electronic reporting.

CPMS (temperature, pressure drop, flow, opacity); calibration, averaging periods, exceedance reporting.

Periodic monitoring (VE, LDAR, inspections).

Parametric/fuel/material analyses; hours/throughput tracking.

## 6.3 Performance Testing

Test plan submittal (methods, locations, QA/QC, notification).

Pre-test calibration and normal ops; test notification; test methods (M1, M2, M3, M4, M5, 7E, 10, 25/25A, etc.).

Final test report within typical 60 days (verify permit).

## 6.4 Recordkeeping

Operational, monitoring, compliance, and administrative records; retain for at least 5 years (Title V).

Make available on request to TCEQ/EPA within permit timelines.

## 6.5 Reporting

Annual Compliance Certification (ACC) signed by RO; identify methods, status, deviations, and certifications.

Semiannual monitoring reports (commonly due Jan 30 and Jul 30).

Deviation/excess emissions reports: prompt notification per permit; written follow-up with details and corrective actions.

NSPS/NESHAP notifications/tests/reports per subpart.

Annual Emissions Inventory via STEERS, typically due March 31 when required.

## 6.6 Permit Modifications and Flexibility

Administrative amendment: non-substantive updates.

Minor revision: does not add or violate applicable requirements or significantly change monitoring/recordkeeping/reporting.

Significant revision: everything else; requires public notice and EPA review.

Operational flexibility and off-permit changes where allowed; notify agencies and consolidate at renewal.

## 6.7 Renewal & Reopenings

Renewal: submit timely and complete application (≥6 months before expiration) to maintain application shield; consolidate changes.

Reopenings: to add new applicable requirements, correct mistakes, or assure compliance (EPA/TCEQ initiated).

## 6.8 Compliance Assurance Monitoring (CAM)

Screen each controlled unit; prepare indicator ranges, QA/QC, frequency, excursion response, and reporting when CAM applies.

# PART 7: DATA & SYSTEMS FOR AI/RAG AUTOMATION

Authoritative corpuses: 30 TAC Ch. 122/116/115/117, 40 CFR part 70/60/63/64/72, TCEQ TV guidance, Chapter 39 notice procedures.

Agency portals: ePermits (Title V), STEERS (EI and e-reporting), EPA CDX/CEDRI (federal reports).

Templates: ACC, Semiannual Monitoring, Deviation, Test Plan/Report.

Facility data: CEMS/CPMS, LDAR DB, fuel/throughput, O&M logs, NSR permit register, change-management/off-permit logs.

Metadata flags in RAG: “federal vs state-only,” due dates, monitoring frequencies, deviation triggers, revision pathways.

# Appendix A: Sample Emissions Summary Table Format

| EPN | Equipment | Pollutant | PTE (tpy) | Allowable (tpy) | Control | Efficiency |
| --- | --- | --- | --- | --- | --- | --- |
| EPN-BOILER1 | NG Boiler (100 MMBtu/hr) | NOx | 43.8 | 43.8 | Low-NOx burner | — |